Nozomi-Core FAT Procedure

PROJECT NAME - 1662666 - Durma-PP13

| Test Case Description |  |  | Expected Result |  | Observed Result |  | Status |  | Comments |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Port Mirroring Configured on Switches: Verify that SPAN ports have been correctly configured on Root Bridge switches and traffic is received on the Nozomi IDS appliance. |  |  | - SSH to root bridge switches and confirm that monitoring sessions for each switch have been configured. - Traffic should be visible in Nozomi web dashboard |  |  |  |  |  |  |
| Licensing: Verify that the Nozomi IDS appliance is properly licensed. |  |  | - Login to Nozomi web interface - Select Administration-> Updates & Licenses - Verify the License Status is OK - Verify a valid license ID is assigned. |  |  |  |  |  |  |
| Verify the appliance is observing Network Traffic |  |  | - On the web GUI, go to Administration > Network Interfaces. Make sure that the Throughput graph is being updated live with network data from all the applicable monitoring interfaces. - On the web GUI, go to Environment > Network view > Traffic tab. Make sure that the graphs are getting populated with live data information. |  |  |  |  |  |  |
| Passive Monitoring Operation: Verify that Nozomi IDS appliance shows assets in the network with correct information. |  |  | On the Network View > Nodes tab, verify that this data is getting populated: • Address • Role • Type • MAC address • Protocols On the Asset view, verify that this data is getting populated: • Name • Address • Role • Type • OS/Firmware |  |  |  |  |  |  |
| Network modeling and visualization operation |  |  | On the Network View > Links tab, verify this data is getting populated: • From • To • Protocol • First Activity • Last Activity • Throughput On the Network View > Sessions tab, verify that this data is getting populated: • Status (review that you have sessions with Status SYN, ACTIVE and CLOSED) • From • To • Protocol • Transport Protocol On the Network View > Graph tab, check that the network graph is being displayed. Apply various filters |  |  |  |  |  |  |
| OT Process Monitoring |  |  | Select Environment section: Process view. Verify the tables are populated with the process variables data of the OT devices talking in the Network: • Host • Host Label • RTU • Name • Protocol • Some fields such as Type, Value, Last FC are applicable only to specific protocols |  |  |  |  |  |  |
| Confirm that the Nozomi IDS appliance is configured in Learning mode. |  |  | - Login to Nozomi web interface - Select Administration-> Security Control Panel-> - Click on “1-Learning” on left-hand pane - Under “Phase Switching”, select “Two Phase” and then “Learning” - Click “Save” |  |  |  |  |  | Note: Nozomi appliance will need to stay in Learning mode for at least 3 weeks once the plant is operating under normal conditions. Then tuning activities will be scheduled with a Nozomi Pro Services engineer before switching to Protected Mode. |
| Test Nozomi ‐ CSH integration |  | CSH should be able to monitor Nozomi physical and virtual devices and should generate an alarm in case of port or device failure. Check the port status in Network Monitor to see if the packet counter changes. |  |  |  |  |  |  |  |
| Confirm time on Nozomi IDS appliance is synced with NTP server |  | The time between NTP server and Nozomi IDS should be the same. - Login to Nozomi web interface - Select Administration-> Date and time - Under NTP section, Verify Enabled check box is selected - Verify IP Address of NTP Server is present (172.16.201.250) Verify that NTP request response packets are observed on the Nozomi IDS using tcpdump. |  |  |  |  |  |  |  |
| Verify that observed assets are assigned to correct zones |  | All the current subnets at the site are registered in Nozomi - PDH - UDH - CyDH |  |  |  |  |  |  |  |
| Verify that Nozomi IDS appliance is configured to forward logs to the McAfee SIEM |  | - Login to Nozomi web interface - Select Administration-> Data Integration - Verify To URI is set to IP Address of McAfee SIEM (ENMELM) is present (172.16.201.103) |  |  |  |  |  |  |  |
| Verify that the Nozomi Dashboard within the McAfee ESM is defined and is populating with log data forwarded from the Nozomi appliance. |  |  |  |  |  |  |  |  |  |
| Restrict Access to Web & SSH interfaces to specific IP Addresses |  |  |  |  |  |  |  |  |  |
| Confirm Firmware version is at least 20.5.1 or newer |  | Check the top banner on main Nozomi Dashboard for installed firmware version. |  |  |  |  |  |  |  |

Installation Specifications

L250 Guardian

| 4.1.1. | Location (Room, Rack, Position) |  |
| --- | --- | --- |
| 4.1.2 | Machine ID |  |
| 4.1.3 | Serial Number |  |
| 4.1.4. | Activation Codes | Base, TI and AI activation codes |
| 4.1.5 | Hostname |  |
| 4.1.6 | Model | L250 |
| 4.1.7. | N2OS version to install |  |
| 4.1.8 | MGMT interface IP address | 192.168.201.70 |
| 4.1.9 | MGMT interface subnet mask | 255.255.255.0 |
| 4.1.10. | MGMT interface duplex and speed (default is auto/auto) | Auto/auto |
| 4.1.11 | Default Gateway IP address | 192.168.201.200 |
| 4.1.12 | DNS server IP address (optional) | 192.168.201.101 |
| 4.1.13. | NTP server IP address (leave empty if syncs to CMC) | 192.168.201.250 |
| 4.1.14 | Monitoring interfaces locations and expected traffic/protocols | PDH, UDH, CyDH |
| 4.1.15. | Monitoring interfaces speed and duplex (default is auto/auto) |  |
| 4.1.16 | Learning Mode configuration | (e.g. Dynamic, 1m) |
| 4.1.17 | CMC token | NA |
| 4.1.18 | CMC IP Address | NA |
| 4.1.19 | Customization lines on n2os.conf.user |  |
| 4.1.20 | Customization commands on web CLI |  |