Nozomi FAT testing

Please include the following tests in the NOZOMI FAT

- Assets management –

  - Show assets management screen

Navigate to Environment -> Asset View

  - Display all detected assets and show how NOZOMI identify them (what description and how is it match to GE description)

Show a controller and a PC details

- New unidentified interface detection –

  - Connect any type of external (not internet connected) interface to one of the monitored networks

Add extra nic to one of the VMs

  - Present how NOZOMI detects this interface and how it notifies this anomaly

Show alerts page

- Incident research –

  - Select past event alerts

Select an alert

  - Present how we can use NOZOMI to check event details

Go to alert details

  - Present how we can use NOZOMI to check for similar past event

– Show group by feature, and filtering based on type, src,dest

- Alerts sorting –

  - Present all current alerts

Show alerts page

  - Sort alerts by categories (Severity, Source address, type, time)

Sort and filter on above categories

  - Present alerts from last 3 days with the same source address

Query created. Can be run to view result.

Show from UI preferable. Then query builder.

- Protocols –

  - Run illegal or unauthorized protocol commands

Change live values. Use unknown protocol that hasn’t been learnt.

  - Present how NOZOMI reacts to those actions.

Show the alert generated.

  - Show details of the command

Show alert details. Unrecognized command like reboot.

  - Show history of detected unauthorized protocol commands.

Sort alerts with the alerts type, or src ip, dest Ip

- Packet capture

  - Capture network traffic for 5 minutes

Trace requested from admin > Other actions > Continuous trace/Custom Trace. Filters can be added for packet capture.

  - Show the captured file

Show in Wireshark

  - Open the captured file and show the data of GE internal protocol  in the different layers to test if all the information was captured

Can show the ports getting used and Ips but not the actual data.

- Use cases for NOZOMI network analysis demo:

- PLC firmware updating.

How are  we going to do this. 	ControlST update and then download

- PLC logic updating.

Download code and show alerts which get generated

- Switching from  security operational mode to unencrypted - manually and automatically in the case of certificate is outdated.

A communication alert between the controller and CA is generated. In secure state no alerts generated related to controller download or variable value changed/forced

- Examples of the detection abnormal network traffic activity. (can be performed with pcap file with abnormal traffic).

SCADA Injection pcap uploaded. After hitting play, alerts are shown.

- Sending operational commands from rogue source.

Does this mean downloading code to controller from external  computer which is not part of HMIs on site

New device with  ControlST etc , sending download command to controller.

- Examples using time machine for cyber event investigation.

time machine -> diff shows newly added nodes and links.

Forced a variable to have different value. In time machine -> diff, the variable tab is still empty.