Nozomi SAT Procedure

| Test Case Description | Expected Result | Observed Result | Status | Comments |
| --- | --- | --- | --- | --- |
| Check that all Nozomi devices are licensed and are shown in the Nozomi CMC. | All Nozomi devices (virtual + physical) show licensed in the web interface. All Nozomi devices (virtual + physical) should show healthy status in CMC. | All devices are shown in CMC. Devices which are not powered up are shown Unreachable currently |  |  |
| Test if creation of SPAN port on physical switches are configured correctly and traffic is received by all Nozomi devices. | SSH to switches connected to Nozomi devices and monitoring session should be configured. Traffic should be visible in Nozomi web interface for all devices | All the switches have been configured for SPAN ports. Traffic is received by Nozomi devices which are powered on. |  |  |

| Test if Nozomi shows assets in the network with correct information. |  | Assets in the network are identified correctly by Nozomi. Name, Type IP information should be correctly displayed although it might not be correct for all devices. | Asset information for some of the devices have been updated to match with their actual asset type. More fine tuning will be required later when the plant is in operation. |  |  |
| --- | --- | --- | --- | --- | --- |
| Test if Nozomi shows vulnerabilities for assets in the network |  | In Nozomi Vulnerabilities menu, CVEs for assets in the network should be shown. |  |  |  |
| Test if proper VM affinity rules are configured on Control Server for HA failover |  | VM affinity rules for Nozomi VMs to not failover to other host should exist |  |  |  |
| Make sure that the Nozomi devices are put in Protecting mode. |  | All the Nozomi devices should show that they are in protecting mode. |  |  |  |
| Connect a laptop with ethernet connection enabled on the PDH network. |  | In the alerts page of Nozomi CMC, we should get an alert for the new device connected. The alert should also be available on Graylog and GE Alarm Viewer. |  |  |  |
| From the connected laptop try to do an SSH to the GE Router. In the alerts page of Nozomi CMC. |  | we should get an alert for the new SSH communication between devices. The alert should also be available on Graylog and GE Alarm Viewer. |  |  |  |
| From an HMI, perform code download to a controller. In the alerts page of Nozomi CMC. |  | we should get an alert for the controller download event. The alert should also be available on Graylog and GE Alarm Viewer. |  |  |  |

| Test Nozomi ‐ CSH integration | CSH should be able to monitor Nozomi physical and virtual devices and should generate an alarm in case of port or device failure. Check the port status in Network Monitor to see if the packet counter changes. | . |  |  |
| --- | --- | --- | --- | --- |
| Test CMC NTP sync with NTP server | The time between NTP server and CMC should be same. Check that request response packets are coming on the CMC using tcpdump. | Primary NTP server is set to NTP1 PDH IP. Secondary NTP device is set as EWS1SVR PDH IP. Time is same between EWS1SVR and CMC. Checked by running tcpdump on CMC and the NTP request response packets are coming. |  |  |
| Test if legitimate operations do not generate alerts | Check-out operations using CMS don’t generate an alerts. There are no weak password alerts. | Alerts from HMI when using CMS have been muted. Alert from BSC-WA1A for network scan has been muted. There is no alert for weak password in the system |  |  |
| Test that all assets are in correct zones | All the current subnets at the site are registered in Nozomi | PDH, UDH, MDH,Customer Network, Ext_Nozomi zones defined. Devices are getting added to the zones. |  |  |