Table of Contents

1.1	Backup Nozomi Physical/Virtual Devices	7

1.2	Checklist for Update Prerequisites/Validation	8

1.3	Identify Nozomi Physical/Virtual Devices Version	9

1.4	Understanding the Nozomi Upgrade Path	10

1.5	VM ESXI v8.0 Compatibility Upgrade	11

1.6	Procedure for Upgrading Nozomi Devices	12

1.7	Updating Threat Intelligence and Asset Intelligence Definitions	14

## Backup Nozomi Physical/Virtual Devices

    - Log into a device that can access the Nozomi web console

    - Log into Nozomi Web Console (Refer to Password Document – Nozomi)

    - Navigate Administration (Gear icon in the Top Right) -> System -> Backup and Restore

    - Under Generate Backup Archive, Select Download button

      - Select Include traces

      - Encryption Password: (Refer to Password Document – Nozomi)

      - Select Download

    - Repeat this process for all Nozomi Devices (Guardian and CMC)

## Checklist for Update Prerequisites/Validation

Note: Ensure all devices (Guardian and CMC) are at the same version

    - Follow instructions in “Backup Nozomi Physical/Virtual Devices” to complete a full backup.

    - Check the Health Page

    - In the Nozomi Web UI, navigate to Gear Icon (Top left of page) -> System -> Health -> Preformance Tab

    - Ensure there are no warning messages and all Health/Services are OK

    - In the Nozomi Web UI, navigate to Gear Icon (Top left of page) -> System -> Health -> Health Log Tab

    - Ensure there are no recent alerts in the health log that could affect the upgrade

    - Failed migrations

    - System under high load

    - X% CPU usage

    - X% RAM used

    - X% disk space used

    - Check Disk for Errors

    - In vSphere, open a web console for Guardian and CMC or Log in using SSH

    - Enter command: zpool status

    - Verify SSH Access

    - Log into the Guardian and CMC using SSH or Putty

    - In a command prompt, enter ssh admin@<IP address for Guardian or CMC>

    - Log in using Nozomi credentials

## Identify Nozomi Physical/Virtual Devices Version

- Log into a device that can access the Nozomi web console

- Log into Nozomi Web Console (Refer to Password Document – Nozomi)

- At the top of the page, a banner will have the version of the Nozomi Device

- Write down the hostname and version of each device

- Repeat this process for all Nozomi Devices (Guardian and CMC)

## Understanding the Nozomi Upgrade Path

- Based on the version list created for current Nozomi Devices on the network,

- Nozomi Upgrade Path:

  - 23.4.0 -> 24.5.1 -> 25.5.0 -> 25.6.0 -> 26.2.0

- Note: Upgrading from 24.5.1 -> 25.5.0 requires the VM to be updated to Host ESXI v8.0 and virtual hardware version 20+. (Refer to VM ESXI v8.0 Compatibility Upgrade Section in this document)

  - If you are lower than the version listed above, update to the next version

    - Ex. If you were at 24.2.0, you would want to upgrade to 24.5.1. Then, follow the rest of the upgrade path.

- If you have a CMC, all updates will be done through the CMC and will be propagated to all connected Guardians.

  - All devices must be in the same upgrade band (Ex. If all Nozomi Device versions are between 24.5.1 and 25.5.0, then that is the same upgrade band)

  - If not, Refer to Individual Nozomi Device Upgrade to get all devices in the same upgrade band before continuing.

- If you do not have a CMC, all Nozomi Devices will have to be individually updated.

## VM ESXI v8.0 Compatibility Upgrade

- Note: Skip this section if you do not have to upgrade from 24.5.1 -> 25.5.0

- Note: Make sure the Backup Section is completed for each Virtual Nozomi Device that needs the ESXI v8.0 upgrade

- For virtual Guardian and CMC,

- Log into the ESXI/vCenter host

- Under Virtual Machines, Select Nozomi Device

- On the VM page, check to see if the compatibility is ESXI v7.x

- Select Shut Down

- Select Actions or Right Click the VM name under Virtual Machines

- Select Upgrade VM compatibility

- Select ESXI 8.0 or ESXI 8.0 U2 virtual machine

- Select Yes

- Select Power on

- Repeat this process for all virtual Nozomi Devices (Guardian and CMC)

## Procedure for Upgrading Nozomi Devices

- Before upgrade follow “Checklist for Update Prerequisites/Validation” section to ensure no errors in upgrade

- Log into the Nozomi CMC or Nozomi Guardian web console

- Note: If the upgrade is performed through CMC, it will update all connected Guardians. If the upgrade is performed on the Guardian, the process will have to be repeated on each Guardian.

- Navigate to Administration -> System -> Operations

- Select the Software Update button

- Note: May look different based on which version upgrading to, but it will have all the same buttons

- Select “Browse”, go to the Prerequisite folder in ISO and select <the corresponding update.bundle> (Ex. “2X.X.X-XXXXXXXX_XXXXX-advanced-update.bundle”.)

- Select Proceed button

- The update may take several minutes to complete, and CMC/Guardian will reboot.

- When finished, Select Close

- Note: If updating through CMC, the update may take 12-20 minutes to propagate the software update to connected Guardians. Then, it will take several minutes to complete, and the Guardian will reboot.

- After update finishes, review “Checklist for Update Prerequisites/Validation” section to ensure no errors in upgrade
Note: If updating from 24.5.1 -> 25.5.0, there may be an error in Web GUI or SSH access. If so, follow the instructions below to fix Web GUI or SSH access issue.

- Log into Guardian/CMC using vSphere web console for the machine that has a Web GUI or SSH access issue

- Enter command: enable-me

- Enter command: setup

- Navigate down using arrow keys to Option 4: Default Router/Gateway

- Navigate up to the empty box and Select Enter Key to Edit setting for Default Gateway

- Enter IP address of DC1

- Navigate down and Select Enter key on OK button

- Navigate up to the Option X: Exit and Select Enter Key

- Enter command: reboot.

- Verify Web GUI and SSH access

- Repeat instructions for Nozomi Guardian/CMC upgrade step till desired update or latest 26.2.0

## Updating Threat Intelligence and Asset Intelligence Definitions

- Note: Update Threat Intelligence and Asset Intelligence through CMC (if applicable). Otherwise follow instructions for updating through Guardian.

- Note: All Nozomi Guardians connected to a CMC will update from the CMC’s Threat and Asset Intelligence version

- Log into the Nozomi CMC/Guardian web console

  - Navigate to System -> Updates & Licenses

  - Select the Update service configuration

    - Select the Manual contents upload

    - Browse for the update files or Drag and Drop them in

    - Wait for the “Verifying….” Message to complete, an “upload succeeded…” message will be displayed.

    - Select “Close”.

    - The version of the update will be displayed in the “Threat Intelligence” window.

- Note: You may need to refresh the browser to see the version of the update.

  - Repeat for “Asset Intelligence” if applicable.